VM/370+

Virtual Machine Facility/370, ESA/390 Edition

Introduction, Installation

and User’s Guide

Release 1  •  Kit Build 5, updated 10 October 2026

based on VM/370 Community Edition V1 R1.2

Publication VMP-0001-0

File No. S370-34

First Edition, revised (October 2026)

This edition applies to VM/370+ Release 1 (kit build 5, 9 October 2026), an ESA/390 conversion of the VM/370 Community Edition V1 R1.2, and to all subsequent builds until otherwise indicated in new editions.

VM/370+ is a hobbyist project. It is not a product of, and this manual is not a publication of, International Business Machines Corporation. IBM, System/370, ESA/390, z/Architecture and VM/370 are names of IBM products and architectures, used here only to identify the systems this software runs on and descends from.

VM/370 itself is in the public domain. The VM/370 Community Edition is maintained by its community. Hercules is an open-source emulator. Linux and Debian are trademarks of their respective owners. cREXX is an open-source REXX implementation.

The project repository holds the update decks, the build tools, the issue register and the test runs from which every statement in this manual can be checked. Comments on this publication may be addressed to the project.

Contents

Preface	1

Chapter 1. Introduction	2

How VM/370+ is built	2

Milestones	2

Chapter 2. Differences from VM/370 Community Edition	4

The machine	4

Virtual machines	4

CP commands	4

CMS	5

Chapter 3. Installing VM/370+	6

What you need	6

Installation steps	6

The first IPL	6

Going back to the Community Edition	7

Chapter 4. Operating VM/370+	8

Start-up and shutdown	8

Storage	8

Rules that matter	8

Chapter 5. Running Linux/390	9

ESA/390 virtual machines	9

What a Linux user needs	9

Starting Linux with the LINUX EXEC	9

Starting Linux by hand	10

The Linux system	11

Several Linux users	11

Linux and CMS side by side	12

Chapter 6. cREXX on VM/370+	13

The 2022 release, built on VM/370+	13

The 24-bit build	13

Cross-compiled programs	13

The current cREXX release	13

Languages written in cREXX	14

Turbo CREXX	14

Chapter 7. Restrictions and Known Problems	16

Appendix A. Command Summary	17

SET ESA	17

IPL	17

LINUX EXEC	17

Appendix B. Contents of the Kit	18

Appendix C. ESA/390 Deviations of the Virtual Machine	19

# Preface

This manual introduces VM/370+, tells you how to install it over a VM/370 Community Edition system running under the Hercules emulator, and describes how to use the two things it adds to VM/370: virtual machines larger than 16 MB with programs that run in 31-bit addressing mode, and virtual machines that run the ESA/390 architecture, in which Linux/390 and a complete Debian system run.

It is written for the VM/370 user who already knows CP and CMS: how to log on, how to use the CP commands of the general user (class G) and of the system operator (class A and B), how to edit and run an EXEC. Where VM/370+ behaves as VM/370 does, this manual says nothing; where it differs, it says how and why.

### How this manual is organized

- Chapter 1, Introduction, says what VM/370+ is and what it is for.

- Chapter 2, Differences from VM/370 Community Edition, lists every externally visible change.

- Chapter 3, Installing VM/370+, takes you from a fresh Community Edition to a running VM/370+ system.

- Chapter 4, Operating VM/370+, covers start-up, shutdown and the rules that keep the system healthy.

- Chapter 5, Running Linux/390, describes ESA/390 virtual machines, the Linux kernel, the Debian system and the network connection to the PC.

- Chapter 6, cREXX on VM/370+, describes the REXX compiler and virtual machine built for and on VM/370+, the languages written in cREXX that run on CMS and on Linux, and Turbo CREXX.

- Chapter 7, Restrictions and Known Problems, lists what does not work yet.

- Appendix A summarizes the new and changed commands, Appendix B lists the files in the installation kit, Appendix C gives the ESA/390 deviations of the virtual machine.

### Conventions

Commands are shown in monospaced type. In command descriptions, uppercase letters are typed as shown and lowercase words stand for values you supply. Examples of terminal sessions are shown in framed figures; lines you type are not marked separately because the context makes clear which they are.

### Summary of changes

This edition is kept up to date as VM/370+ grows; each change is listed here, newest first.

- 10 October 2026. Regina REXX and THE under Debian; languages written in cREXX for CMS and Linux (BASIC, SNOBOL4, Pascal, Lisp, Smalltalk, PL/M, Prolog, LOGO); Turbo CREXX on both; four more minidisks for CMSUSER (196–199); the native build of the current cREXX (M8.2); Python and GNU C under Debian; milestones brought up to date.

- 9 October 2026. First edition, for kit build 5.

PC means the personal computer on which Hercules runs. Real storage and devices are those of the Hercules machine; virtual storage and devices are those CP gives a user.

# Chapter 1. Introduction

VM/370 was written for System/370: 24-bit addresses, 16 MB of storage, S/370 channel I/O. VM/370+ is the VM/370 Community Edition V1 R1.2 with its Control Program (CP) converted to the ESA/390 architecture. It runs on Hercules in ARCHMODE ESA/390 and gives its users:

- Virtual machines up to 256 MB. CP builds ESA/390 segment and page tables, runs in 31-bit addressing mode itself, and gives a virtual machine as much storage as its directory entry allows.

- 31-bit programs under CMS. An EC-mode CMS, IPLed from MAINT’s 290 disk, lets programs obtain storage above 16 MB (the HIGHSTOR facility) and run in 31-bit addressing mode, with a C compiler and library for such programs.

- ESA/390 virtual machines. SET ESA ON makes a virtual machine an ESA/390 machine: channel subsystem I/O (SSCH, TSCH, MSCH, STSCH), the ESA/390 PSW and control registers, home and primary address spaces, IEEE floating point. Linux/390 runs in it.

- Linux and Debian. A Linux 4.0 kernel (31-bit, the last Linux release that supports 31-bit machines) IPLs from the virtual card reader and runs Debian 7 from a 3390 disk, with SSH access from the PC over a channel-to-channel adapter.

- cREXX. The cREXX REXX compiler, assembler and virtual machine run on VM/370+ and are built on it.

Everything else is VM/370: the same CP commands, the same directory, the same CMS for 24-bit work, the same spooling, the same operator. Programs that ran under the Community Edition run unchanged.

## How VM/370+ is built

VM/370+ is a set of UPDATE decks applied to the Community Edition source, assembled on the system itself, and loaded into the CP nucleus with the standard VMFLOAD procedure. The repository of the project holds every deck, the generator that produces them, the build tools, the issue register and the regression runs. Nothing in VM/370+ is a binary patch.

## Milestones

The conversion was done in milestones, each closed by a run that exercises it. Figure 1 shows where they stand.

| Milestone | Content | Status |
| --- | --- | --- |
| M0–M1 | ESA/390 instructions for the assembler; CP IPLs in ESA/390 mode, console and channel subsystem | complete |
| M2 | ESA/390 DAT tables, CP in 31-bit mode, 31-bit guests | complete |
| M3–M4 | CMS under the new CP, shared CMS by page frame, several users, 4 KB storage keys | complete |
| M4b | real storage up to 2 GB (above 16 MB as CP’s paging store) | complete |
| M4b.3 | CP uses real frames above 16 MB directly | planned |
| M5 | EC-mode CMS with storage above 16 MB (HIGHSTOR), 31-bit C (cross and native) | complete |
| M6 | a CMS nucleus that is itself 31-bit | planned, before M9 |
| M7 | ESA/390 virtual machines, Linux/390, Debian, SSH | complete |
| M7.8 | Linux for several users from a linkable disk (LINUX EXEC); Linux and CMS side by side | complete; two Debians at once need M4b.3 |
| M7.9 | channel programs of any length for ESA/390 guests (no 64-CCW limit) | planned |
| M7.10 | IPL of Linux from its disk (zipl) and a CP name LINUX | next |
| M8 | cREXX: the 2022 release built natively (M5); the current release cross-built (M8.1) | complete |
| M8.2 | the current cREXX built natively with GCC380 | compiler, assembler and VM work; the library does not build yet |
| — | languages in cREXX for CMS and Linux; Turbo CREXX | Linux complete; CMS: all but Smalltalk |
| — | GNU C 4.6, Python 2.7 and 3.2, Regina REXX and THE under Debian | complete |
| M9 | z/Architecture (64-bit) | planned, after all of the above |

Figure 1. VM/370+ milestones

# Chapter 2. Differences from VM/370 Community Edition

## The machine

| Item | Community Edition | VM/370+ |
| --- | --- | --- |
| Hercules architecture | ARCHMODE S/370 | ARCHMODE ESA/390 |
| CPU model | 4381, ECPS:VM available | 3090, ECPS:VM not used (its assists are S/370-only) |
| Hercules main storage | 16 MB | 64 MB in the kit; up to 2048 MB |
| CP’s addressing mode | 24-bit | 31-bit |
| Real storage used by CP | up to 16 MB | 16 MB as frames; the rest as a fast paging store |
| I/O | S/370 channels (SIO, TIO, HIO) | ESA/390 channel subsystem (SSCH, TSCH, HSCH) |
| Storage keys | 2 KB keys | 4 KB keys (ISKE, SSKE, RRBE) |
| Segment size | 64 KB | 1 MB (ESA/390 has no other) |

Figure 2. The real machine

## Virtual machines

- Storage. A virtual machine may have up to 256 MB. The directory’s maximum (the second size on the USER statement) is checked as before; the kit’s directory allows 256 MB for CMSUSER and 64 MB for MAINT.

- Shared CMS. The CMS saved system is shared by page frame rather than by segment: with 1 MB segments, sharing whole segments would expose private CMS storage to other users. The effect for the user is none.

- 31-bit virtual machines. A virtual machine in EC mode may run with PSW bit 32 on (31-bit addressing); CP translates its addresses in 31 bits.

- ESA/390 virtual machines. New command SET ESA ON|OFF; see Chapter 5 and Appendix A.

- More disk space for CMSUSER. Besides 191–195 on VM50U0, CMSUSER has four more minidisks, 196–199, of 115 cylinders (about 52 MB) each on VM50-8, in the cylinders the Community Edition leaves free there. They come formatted, labeled CMS196 to CMS199: ACCESS 196 H (197 I, 198 J, 199 K). A CMS minidisk on a 3350 holds at most 65,535 blocks of 800 bytes, about 52 MB; larger minidisks come with milestone M6.

## CP commands

| Command | Change |
| --- | --- |
| SET ESA ON¦OFF | New (class G). ON makes the next IPL of a device an IPL of an ESA/390 machine; OFF returns to System/370 at once. |
| IPL cuu | A user who is running a shared named system (CMS) is IPLed with CLEAR, so that no copy of the shared pages stays in the virtual machine. |
| IPL name | Always IPLs a System/370 machine; a pending SET ESA ON waits for the next IPL of a device. |
| DEFINE STORAGE | Up to 256 MB. Leaves the named system the user was running. |
| DISPLAY, STORE | Address storage above 16 MB. |
| QUERY VIRTUAL STORAGE | Shows five digits only (131072K appears as 31072K); DEFINE STORAGE echoes the right value. |

Figure 3. Changed CP commands

## CMS

- The ordinary CMS (IPL CMS, IPL 190) is unchanged: a System/370 BC-mode CMS with 16 MB of addressable storage.

- An EC-mode CMS is on MAINT’s 290 disk (IPL 290). With it, the HIGHSTOR nucleus extension (loaded by the system profile) manages storage from 16 MB to the end of the virtual machine; SVC 120 (GETMAIN/FREEMAIN RU) gives programs storage above the line.

- GCC380, the AMODE 31 build of the Community Edition’s GCC, runs above 16 MB; GCCLIB31 is a C library for programs that run in 31-bit mode with their heap above 16 MB.

- CPWATCH is not autologged: it is an S/370 monitor that loops under this CP. The two lines in AUTOLOG1’s PROFILE EXEC are commented out.

# Chapter 3. Installing VM/370+

## What you need

- A fresh, unmodified extraction of VM370CE_V1_R1_2.zip. Do not install VM/370+ over a system you care about.

- Hercules 4.x (SDL Hyperion; tested with 4.7 and 4.9.1) or Hercules 3.13. Programs cross-compiled for 31-bit mode need Hercules 4.x, which has the z900-level instructions in ESA/390 mode.

- A 3270 terminal emulator; the wc3270 in the Community Edition package is sufficient.

- For Linux networking on Windows: the CTCI-WIN package for SDL Hercules (TunTap64.dll, with Npcap). On Linux or WSL2: the tun driver, and Hercules running as root or with hercifc installed setuid.

- The VM/370+ kit: seven zip files, each under 30 MB.

| Part | Contents |
| --- | --- |
| 1 | vm370ce.conf (ESA/390), the shadow files of the system volumes (CP nucleus, CMS, directory), README, tools |
| 2 | disks/vm50-4.cckd: VM50-4 with its paging area record corrected |
| 3 | the shadow file of the CMSUSER volume |
| 4 | vm370plus/linux/: the Linux kernel and parameter decks, the CMS files for MAINT 19D, kernel configuration and patches |
| 5–7 | the Debian disk, disks/lnx190.cckd, in three pieces |

Figure 4. The installation kit

## Installation steps

- Unzip VM370CE_V1_R1_2.zip into a new directory.

- Unzip kit parts 1 to 7 over it, replacing files when asked.

- Join the Debian disk. In the Community Edition directory, at a Windows command prompt:

copy /b disks\lnx190.cckd.001+disks\lnx190.cckd.002+disks\lnx190.cckd.003 disks\lnx190.cckd del disks\lnx190.cckd.00?

Figure 5. Joining the Debian disk (Windows)

On Linux or WSL2 use cat disks/lnx190.cckd.00? > disks/lnx190.cckd. The file disks/lnx190.sha256 holds the checksum of the joined file. Keep a copy of lnx190.cckd: it is your Linux system disk and Hercules writes to it.

- If you want the Linux network, remove the # in front of the line #0600.2 CTCI 10.1.1.2 10.1.1.1 in vm370ce.conf.

- Start the system as you start the Community Edition: vm370ce.cmd on Windows, vm370ce.sh elsewhere.

## The first IPL

The Community Edition’s hercules.rc IPLs from 6A1 by itself when Hercules starts. The first time, CP has no warm-start data and asks for the start type; answer /cold. When the console shows DMKCPI966I Initialization complete, the system is up.

HHCCP041I SYSCONS interface active VM/370+ Online Start ((Warm¦Force¦COLD¦CKPT) (DRain) (DIsable) (NOAUTOlo)) or (SHUTDOWN) /cold NOW 09:51:57 GMT FRIDAY 10/09/26 DMKCPI966I Initialization complete

Figure 6. The first IPL of VM/370+

Attention: Do not type ipl 6a1 again once CP has initialized: it resets the running system. Type it yourself only if nothing has been IPLed (no DMKCPI messages at all).

Then connect the 3270 emulator to localhost:3270 and log on as usual.

## Going back to the Community Edition

Delete disks/shadows/*_1.shadow and restore vm370ce.conf and disks/vm50-4.cckd from the Community Edition zip.

# Chapter 4. Operating VM/370+

## Start-up and shutdown

Operation is that of VM/370. The operator’s console is 0009; ENABLE ALL brings up the 3270 terminals. To stop the system, the operator types shutdown on 0009 and then exit at the Hercules console.

Attention: Always end with shutdown at the operator’s console before you leave Hercules. Killing Hercules leaves the shadow files inconsistent. Never run two Hercules instances on the same disks.

## Storage

CP’s own machine is 16 MB. With MAINSIZE above 16 in vm370ce.conf (the kit has 64), CP uses the storage above 16 MB as a paging store that no channel addresses: a page-out copies a frame there, a page-in copies it back. Up to 2048 MB has been tested. Virtual machines may be up to 256 MB.

Note: Because CP keeps its working frames in the low 16 MB, a 64 MB Linux machine pages heavily, and while one runs, other users’ work is slower. Using frames above 16 MB directly (milestone M4b.3) is planned.

## Rules that matter

- A user who has IPLed the shared CMS may IPL another system; the shared pages are dropped first (the IPL is done with CLEAR). The shared-system models stay in storage until CP is IPLed again.

- CP emulates the System/370 interval timer from the TOD clock, since ESA/390 has none; time slices end and virtual interval timers run.

- CPWATCH is not autologged (see Chapter 2).

# Chapter 5. Running Linux/390

This chapter describes ESA/390 virtual machines and Linux/390 in them. Linux runs in two forms: a small system in storage (BusyBox), which needs no disk, and Debian 7 on a 3390 disk of its own.

## ESA/390 virtual machines

SET ESA ON makes a virtual machine an ESA/390 machine at its next IPL of a device. Such a machine has:

- the ESA/390 PSW and control registers, primary, secondary and home address spaces, and ESA/390 DAT, which CP shadows;

- the channel subsystem: its devices are subchannels, I/O is started with SSCH and completed with TSCH; channel programs may use format-0 or format-1 CCWs and 31-bit IDAWs;

- IEEE binary floating point, the CPU timer, the clock comparator and SIGP for its one processor.

IPL CMS (or any IPL by name) gives a System/370 machine again; the pending SET ESA ON then applies to the next IPL of a device. SET ESA OFF returns to System/370 at once. Appendix C lists where the virtual machine deviates from the ESA/390 architecture.

## What a Linux user needs

| Resource | Virtual address | How it is provided |
| --- | --- | --- |
| Storage | 64 MB | DEFINE STORAGE 64M, or 64M in the directory |
| Console | 009 (3215) | the terminal; Linux uses it in line mode |
| Card reader | 00C | the kernel and the parameters are IPLed from it |
| Debian disk | 250 (3390) | a real 3390, attached or dedicated to the user |
| Network | 620 and 630 (CTCA) | a CTC pair, attached or dedicated to the user |

Figure 7. Resources of a Linux virtual machine

The operator attaches the devices:

attach 600 to maint as 620 attach 601 to maint as 630 attach 190 to maint as 250

Figure 8. Attaching a Linux user’s devices (operator)

Each device must be on its own virtual control unit (620 and 630, not 620 and 621). For a permanent arrangement put DEDICATE statements in the user’s directory entry.

## Starting Linux with the LINUX EXEC

The Linux kernel, the IPL parameter files and the LINUX EXEC are on MAINT’s 19D disk, which every CMS user accesses as the U disk. From CMS, with 64 MB of storage and the devices attached, type LINUX:

logon cmsuser cp define storage 64m ipl cms Ready; linux LINUX: punching the kernel and CMSUSER LXPARM to your reader ... PUN FILE 0002 TO CMSUSER COPY 01 NOHOLD PUN FILE 0003 TO CMSUSER COPY 01 NOHOLD LINUX: IPL 00C as an ESA/390 machine Linux version 4.0.0+ ... ... root: the system on /dev/dasda1 -- switch_root INIT: version 2.88 booting ... Debian GNU/Linux 7 vm370plus ttyS0 vm370plus login:

Figure 9. Starting Linux from CMS

The EXEC holds the files already in your reader, punches the two pieces of the kernel into your own reader as one spool file and then your parameter file, sets ESA ON and IPLs the reader. The parameter file is userid LXPARM on the U disk, or DEFAULT LXPARM if there is none; it carries your IP address. Afterwards CP CHANGE RDR ALL NOHOLD releases your other reader files.

| File on MAINT 19D | Contents |
| --- | --- |
| LINUX48 KERNEL1, KERNEL2 | the kernel with its initramfs, 91,696 card images in two files (a CMS file holds at most 65,533 records) |
| LINUX EXEC | the EXEC described above |
| DEFAULT LXPARM | no_removal_warning conmode=3215 condev=0x0009 |
| userid LXPARM | the same plus vmip= and vmpeer=, the user’s address and the PC’s end |

Figure 10. Linux files on MAINT 19D

## Starting Linux by hand

Without the EXEC, the operator spools the two decks from vm370plus/linux/ to the user’s reader, and the user IPLs it:

/cp spool 00c class * devinit 00c vm370plus/linux/linux48.rdr ebcdic eof /cp start 00c devinit 00c vm370plus/linux/lxparm.rdr ebcdic eof /cp start 00c

Figure 11. Spooling the Linux decks (Hercules console)

logon maint cpcms noipl cp define storage 64m cp set esa on cp ipl 00c

Figure 12. IPL of Linux from the reader

The ID card of the two decks names MAINT; for another user change the first card or spool the decks with /cp spool 00c ... and TRANSFER. IPL from the reader consumes the files: spool them again for the next IPL.

## The Linux system

### Boot sequence

The kernel’s initramfs (BusyBox) brings the 3390 online, creates a partition and a file system on a new disk, groups the CTC pair as ctc0 and configures it from the IPL parameters (default 10.1.1.2, peer 10.1.1.1). If the disk holds a system (/sbin/init), it hands over to it with switch_root; otherwise it stays in storage with the prompt vm370plus:~#. The parameter vmroot=ram (deck lxparmram.rdr) keeps the in-storage system even when Debian is on the disk, which is how you repair the disk.

### Debian

The disk holds Debian 7.11 (wheezy) for s390: sysvinit, udev, rsyslog, cron, OpenSSH, apt and dpkg, root on ext2, a 128 MB swap file. Log in as root with password vm370plus, and change the password.

Development tools are installed: GNU C and C++ 4.6.3 with make, Python 2.7.3 (python) and Python 3.2.3 (python3), Regina REXX 3.6 (regina or rexx, classic REXX as on CMS) and THE 3.3, The Hessling Editor (the, an XEDIT-like editor; use it over SSH, since the 3215 console cannot show a full screen). cREXX and the languages of Chapter 6 are in /usr/local/bin.

root@vm370plus:~# cat /etc/debian_version 7.11 root@vm370plus:~# df -h / Filesystem Size Used Avail Use% Mounted on /dev/dasda1 771M 325M 407M 45% / root@vm370plus:~# free total used free shared buffers cached Mem: 60512 21688 38824 0 2220 13792

Figure 13. Debian on VM/370+

### The console

The 3215 is a line device. Programs that use the full screen (vi, top without -n1) draw poorly; use them only if you must. Ctrl-C cannot be typed: type ^c and Enter instead (the 3215 driver turns ^c, ^d and ^z into the control characters). When CP shows MORE..., press CLEAR or PA2.

### Network and SSH

Linux talks to the PC over a channel-to-channel adapter that Hercules connects to a tun interface on the PC (CTCI). From the PC: ssh root@10.1.1.2, or PuTTY to host 10.1.1.2, port 22. The first login after a boot may take a minute while the SSH server gathers randomness. Internet access from Linux needs routing and NAT on the PC; /etc/apt/sources.list points at archive.debian.org.

### Stopping Linux

poweroff (or halt) stops Debian; CP then shows a disabled wait. CP LOGOFF releases the attached devices. IPL CMS returns to CMS in the same session.

## Several Linux users

Each Linux user needs his own copy of the Debian disk, his own CTC pair with a CTCI line in vm370ce.conf and his own IP address in userid LXPARM. The kit’s example: MAINT on 600/601 and 190, address 10.1.1.2; CMSUSER on 602/603 and 191, address 10.1.2.2.

0600.2 CTCI 10.1.1.2 10.1.1.1 0602.2 CTCI 10.1.2.2 10.1.2.1 0190 3390 disks/lnx190.cckd 0191 3390 disks/lnx191.cckd

Figure 14. Two Linux users in vm370ce.conf

Make lnx191.cckd a copy of the Debian disk. The disks are identical: the address comes from the IPL parameters, not from the disk. Give Hercules enough storage for CP’s paging store: MAINSIZE 256 (the kit’s value) for two Linux users, more for more. With too little, CP fills its DASD paging space (DMKPGT400I SYSTEM TEMP SPACE FULL) and the Linux machines fail.

Attention: Today one Debian system at a time is practical. Two Debian systems run correctly side by side, but CP keeps its frames in 16 MB of real storage, and two Debian working sets (about 13 MB and 7 MB) do not fit: the scheduler keeps one machine waiting for storage, and it barely moves. Using frames above 16 MB (milestone M4b.3) removes this limit. A Debian system and small Linux systems in storage, or a Debian system and any number of CMS users, run together without problems.

## Linux and CMS side by side

Linux users and CMS users share the system as any two VM/370 users do. Tested: one user runs Debian and writes and reads files under load, while another user works in CMS (LISTFILE, COPYFILE, TYPE, ERASE), defines 64 MB, starts Linux with LINUX, stops it with poweroff and goes back to CMS with IPL CMS; the Debian system’s files are unchanged throughout.

# Chapter 6. cREXX on VM/370+

cREXX is a REXX implementation built as a compiler (RXC), an assembler (RXAS), a disassembler (RXDAS) and a virtual machine (RXBVM, RXVM). It needs more than 16 MB to compile larger programs, which is where VM/370+ came from.

## The 2022 release, built on VM/370+

CMSUSER’s disks hold the 2022 cREXX tree (F0041). In the EC-mode CMS it is compiled by GCC380 with GCCLIB31 and runs in 31-bit mode with its heap above 16 MB:

logon cmsuser cp define storage 256m cp link maint 290 290 rr ipl 290 exec crxmk31 ... rxcn basic rxasn basic rxbvmn basic 0.1 Ready;

Figure 15. Building and running cREXX on VM/370+

CRXMK31 EXEC compiles the tree and links RXCN, RXASN, RXBVMN and RXDASN (about 90 seconds of CPU time). GCLB31 EXEC builds the GCCLIB31 library itself. The output equals that of the cross-compiled build line for line.

## The 24-bit build

The same tree also builds with GCC370 as a 24-bit program (rxc, rxas, rxdas, rxbvm on CMSUSER’s E disk); in the EC-mode CMS with global txtlib gcclib, rxbvm -l d ascommon and rxbvm -l d asebcdic run their test sets.

## Cross-compiled programs

rxc31, rxas31 and rxbvm31 on CMSUSER’s A disk were compiled on the PC with GCC 13 (-m31), newlib and a small CMS runtime (vm370plus/m5f/). They need Hercules 4.x.

ipl 290 rxc31 basic rxas31 basic rxbvm31 basic libctest a b

Figure 16. Cross-compiled cREXX

## The current cREXX release

The current release (1.0.0-beta.3) needs a C99 compiler; GCC380 is GCC 3.2.3. It runs on VM/370+ cross-compiled with GCC 13 (RXC8, RXAS8, RXBVM8, milestone M8.1). RXC8 needs LIBRARY RXBIN and RXCEXITS RXBIN on the disk named by -l; source files are fn CREXX:

rxc8 -l a -i a hello rxas8 -l a hello rxbvm8 -l a hello Hello from current cREXX on VM/370+ sum 1..10 = 55 Ready;

Figure 17. The current cREXX on CMS

Milestone M8.2 builds the same release natively with GCC380 and GCCLIB31 (RXC82, RXAS82, RXBVM82). The three work for programs that call no library functions; the library itself does not build natively yet (Chapter 7). Until it does, use the M8.1 tools. Native cREXX on CMS runs in EBCDIC (IBM-1047) throughout, as IBM REXX does.

## Languages written in cREXX

Several languages are implemented as cREXX programs. They are compiled once on the PC and run unchanged on CMS (through RXBVM8) and on Linux (through rxvm). Floating point comes from rxfloat, a cREXX version of the cREXX float plugin, so no native plugin is needed.

| Language | CMS | Linux | Notes |
| --- | --- | --- | --- |
| BASIC | BASIC fn.bas | basic fn.bas | classic, MS and ANSI dialects (-MS, -FB, -ANSI) |
| SNOBOL4 | SNOBOL fn.sno | snobol fn.sno | patterns, tables, arrays |
| Pascal | PASCAL fn | pasc fn.pas | compiles Pascal to cREXX, then runs it |
| Lisp | LISP fn.lisp | lisp fn.lisp | a Common Lisp subset; without a file, a read-eval-print loop |
| Smalltalk | SMALLTLK fn.st | smalltalk fn.st | Smalltalk-80 with GNU Smalltalk bracket syntax; class library in *.ST |
| PL/M | PLM fn | plmc fn.plm | PL/M-80 compiled to cREXX; ( COMPILE or -c compiles only |
| Prolog | PROLOG | prolog | consult files with consult('family.pl')? |
| LOGO | LOGO fn.logo out | logo fn.logo out.svg | turtle graphics written as SVG |

Figure 18. Languages in cREXX

On CMS the languages arrive as one reader deck (LANGS); READCARD * puts the files on your A disk and LANGSUP EXEC rebuilds the modules from their hex form. Use one of CMSUSER’s large disks as A for it (for example ACCESS 196 A). CMS cuts a command argument to eight characters, so the EXECs pass file names through the program stack: type the names as usual.

On Debian the languages are part of the cREXX bundle (lxcrexx.tgz), unpacked under /usr/local; the examples are in /usr/local/share/language.

## Turbo CREXX

Turbo CREXX is a small Turbo-Pascal-style front end: a menu that creates, edits, compiles and runs one cREXX program at a time. It does not implement REXX; it calls the real cREXX tools.

| Key | Action |
| --- | --- |
| N | new program (from a template), then edit it |
| O / E | open an existing program; edit it |
| C | compile: compiler and assembler, no run |
| R | compile and run |
| L | copy an example (hello, fibfact) to your disk or directory |
| D | show the versions of the tools |
| Q | quit |

Figure 19. Turbo CREXX menu

On Debian, type turbocrexx (or turbocrexx file.crexx). The editor is $EDITOR, else nano or vi. Underneath, crexx file.crexx compiles and runs a program (crexx -noexec only compiles): a small driver of VM/370+ for rxc, rxas and rxvm, since the upstream crexx driver is not yet built for s390.

On CMS, type TURBO (or TURBO fn). Programs are fn CREXX A; the editor is EDIT; compile and run use RXC8, RXAS8 and RXBVM8 with LIBRARY RXBIN on A. Without the menu: TURBO C fn, TURBO R fn, TURBO E fn. The examples are HELLO TCEXAMPL and FIBFACT TCEXAMPL.

turbo T U R B O C R E X X --- VM/370+ CMS Current file: (none) N New O Open E Edit C Compile R Run L Examples D Versions Q Quit Command:

Figure 20. Turbo CREXX on CMS

# Chapter 7. Restrictions and Known Problems

| Area | Restriction |
| --- | --- |
| Real storage | CP’s frames are in the low 16 MB; storage above is a paging store. Large guests page heavily. |
| Linux disk I/O | A format-1 channel program is converted to at most 64 format-0 CCWs; Linux limits its disk requests to 128 KB for that reason. |
| Shared systems | The models of a shared named system are not freed when the last user leaves; they stay until CP is IPLed. |
| CP dump | DMKDMP cannot write its dump under ESA/390 (I-201); after an abend the registers must be taken from Hercules. |
| CMS | The CMS nucleus is 24-bit; programs above 16 MB use HIGHSTOR and SVC 120 (milestone M6 will change this). |
| CPWATCH | Loops under this CP; not autologged. |
| QUERY VIRTUAL STORAGE | Shows five digits. |
| Multiprocessing | One CPU only. |
| cREXX, native | The natively built current cREXX (M8.2) cannot build its library yet; use RXC8, RXAS8 and RXBVM8 (M8.1). |
| CMS file names | Programs cross-compiled for CMS cut a file name longer than eight characters to eight (Smalltalk’s collections.st is COLLECTI ST). |
| Linux | Linux 4.0 31-bit is the last kernel with 31-bit support; Debian 7 is the last Debian for s390 (31-bit). No IPL of Linux from DASD yet. |

Figure 21. Restrictions

# Appendix A. Command Summary

## SET ESA

SET ESA ON SET ESA OFF

Figure 22. SET ESA

Class G. ON: the next IPL of a device (not an IPL by name) makes the virtual machine an ESA/390 machine; it stays pending across IPL CMS. OFF: the machine is a System/370 machine at once and nothing is pending. Any operand other than OFF means ON.

## IPL

As in VM/370, with two changes: an IPL by name always gives a System/370 machine; an IPL of a device by a user who runs a shared named system is done with CLEAR.

## LINUX EXEC

LINUX

Figure 23. LINUX EXEC

On MAINT 19D. Punches LINUX48 KERNEL1, KERNEL2 and userid LXPARM (or DEFAULT LXPARM) to the user’s reader, holding the other reader files, and IPLs the reader as an ESA/390 machine. Needs 64 MB and, for Debian, the devices listed in Chapter 5 under “What a Linux user needs”.

# Appendix B. Contents of the Kit

| Path | Contents |
| --- | --- |
| vm370ce.conf | ESA/390 configuration, MAINSIZE 64, the CTC and 3390 lines for Linux |
| disks/shadows/*_1.shadow | the VM/370+ system: CP nucleus, CMS, directory, user disks |
| disks/vm50-4.cckd | VM50-4 with the paging area record corrected |
| disks/lnx190.cckd | the Debian disk (after joining parts 5–7) |
| README-VM370PLUS.txt | the short form of this manual |
| vm370plus/linux/ | linux48.rdr, lxparm.rdr, lxparmram.rdr, kernel configuration, patches, initramfs sources, the Debian post-install script |
| vm370plus/m5f/ | the CMS runtime for cross-compiled 31-bit C programs |
| vm370plus/*.EXEC, *.C | GCC31 EXEC, CRXMAKE EXEC, examples |

Figure 24. Kit contents

# Appendix C. ESA/390 Deviations of the Virtual Machine

The ESA/390 virtual machine is complete enough for Linux. Where it deviates from the architecture, the deviation is recorded in the project’s register (documents 39 and 40). The ones a user may notice:

- A format-1 channel program is converted to format 0, at most 64 CCWs per start.

- No SCLP and no z/VM interfaces (DIAGNOSE codes of z/VM); Linux must be told conmode=3215 condev=0x0009.

- One processor; SIGP is answered for that processor only.

- Selector channels of an ESA/390 guest behave as block multiplexer channels.

- Linux’s machine-check handling is not exercised: CP presents no machine checks to the guest.